Playground Audit/Inspection

District:   __________________________________________________________________

School:  ___________________________________________________________________

Play Site:   _________________________________________________________________

Date of Inspection:   _________________________________________________________

Inspector(s):  _______________________________________________________________

Age Appropriate Designation (circle one)     Toddler (0-2 yrs)	Pre-School (2-5 yrs)	School Age (5-12)							      Multiple and separated

Provided by school site/district (Y/N):		Inventory of playground equipment ______

Diagram of play area______		Diagram of composite play structures_______

| C | NC | N/A | SURFACING MATERIAL | Comments |
| --- | --- | --- | --- | --- |
|  |  |  | Surfacing material is free of debris, weeds, and standing water. |  |
|  |  |  | Area is free of trip hazards |  |
|  |  |  | All anchoring posts and devices are below material |  |
|  |  |  | Loose-fill surfacing material: |  |
|  |  |  | Fall material is a minimum of 9 inches deep |  |
|  |  |  | Fall material is evenly distributed (i.e. it is not piling up under platforms, etc.) |  |
|  |  |  | Fall material is loose and ample |  |
|  |  |  | Unitary surfacing material: |  |
|  |  |  | Surfacing material is in good condition |  |

Type of fall material:    ____________________________________________________________________________

Additional comments regarding fall material:  _________________________________________________________

_______________________________________________________________________________________________

School:__________________________	     Play site: _______________________________ 	Date: ___/____/____

| C | NC | N/A | USE ZONES | Comments |
| --- | --- | --- | --- | --- |
|  |  |  | Use zone around stationary equipment is 72” - Adjacent play structures - 30” or less can be 6’; more than 30” must be 9’ |  |
|  |  |  | Use zone in front of slides is minimum 6’ – maximum 8’ - Slides greater than 6 feet, use zone in front should be as long as the slide is tall up to a max of 8 feet. - Use zone shall never overlap another use zone |  |
|  |  |  | Use zone for rotating play equipment is 72” - Use zone may not overlap any other equipment |  |
|  |  |  | Use zone for swings is twice the height of surface material to the pivot point - Front-to-rear use zone cannot overlap any other equipment - Use zone 6 feet from support posts - Use zone of support structure for adjacent swings may overlap |  |
|  |  |  | Use zone for bucket seat swings is twice the height from seat to the pivot bar - Use zone 6 feet from support posts |  |
|  |  |  | Use zone for multi-axis swings is the height of the swing seat to the pivot point plus 72” - Use zone 6 feet from support posts - Use zone cannot overlap any other equipment - Use zone of support structure for adjacent swings may overlap |  |
|  |  |  | Use zone for rocking/springing play equipment - 72” if user is intended to sit - May overlap when each structure consists of a seat with height of 30” or less |  |
|  |  |  | Use zone for rocking/springing play equipment - 84” if user is intended to stand - Use zone may not overlap any other equipment |  |
|  |  |  | Use zone for a track ride shall extend no less than 72” in all directions |  |
|  |  |  | Overhead obstructions within the use zones are 84” above designated playing surfaces and pivot of swings. |  |

School:__________________________	     Play site: _______________________________ 	Date: ___/____/____

| C | NC | N/A | ACCESSIBILITY |  | Comments |
| --- | --- | --- | --- | --- | --- |
|  |  |  | Container box has accessible entrance of at least (60”) clear width - No change in level of greater than ¼” delta or ½” beveled along path of travel at ramp and entrance to the play area. - Maximum ramp slope is 5% (1:12) into the play area |  |  |
|  |  |  | Accessible route - 60” minimum clear width within container box - 60” minimum clear width outside container box but may be narrowed to 36” for a distance of 60” - 80” vertical clearance along ground level accessible route |  |  |
|  |  |  | Transfer platform - 11” to 18” above the protective surfacing - Minimum 24” wide - Minimum 14” deep - Transfer support provided - May be primary access if play area has less than 20 elevated play components - 48” square area adjacent to platform for turning/parking |  |  |
|  |  |  | Transfer steps: - 24” wide minimum - 14” deep minimum - 8” high maximum - Contrast striping that is 2” wide and 1” maximum away from the edge |  |  |
|  |  |  | Play tables: - 24” high minimum knee clearance - 31” maximum height of table - 30” wide minimum - 17” deep minimum |  |  |
|  |  |  |  | Accessible play components: |  |  |
|  |  |  |  | - Ground level (see table on next pg for # required) |  |  |
|  |  |  |  | - Elevated – at least 50% of total elevated |  |  |

ACCESSIBLE PLAY COMPONENT REQUIREMENTS

| Number of elevated play components provided | Minimum number of ground-level play components required to be on accessible route | Minimum number of different types of ground-level play components required to be on accessible route |
| --- | --- | --- |
| 1 | Not applicable | Not applicable |
| 2 to 4 | 1 | 1 |
| 5 to 7 | 2 | 2 |
| 8 to 10 | 3 | 3 |
| 11 to 13 | 4 | 3 |
| 14 to 16 | 5 | 3 |
| 17 to 19 | 6 | 3 |
| 20 to 22 | 7 | 4 |

|  |  |  | If playground has 20 or more elevated play components: |  |
| --- | --- | --- | --- | --- |
|  |  |  | Elevated accessible routes: - 36” wide - Ramp - handrail height between 20” and 28” |  |
|  |  |  | Landings - Wide as the ramp they connect to - Minimum length 60 inches to allow for direction change |  |

School:__________________________	     Play site: _______________________________ 	Date: ___/____/____

| C | NC | N/A | STAIRWAYS, LADDERS AND HANDRAILS | Comments |
| --- | --- | --- | --- | --- |
|  |  |  | Rung ladders, climbing nets, arch climbers, tire climbers are not used as the sole access on equipment intended for pre-school age children - Flexible access components for 2-5 years allows user to bring both feet to same level before ascending to next level |  |
|  |  |  | Flexible components used for access are securely connected at both ends - Anchoring device must be beneath the ground surface material |  |
|  |  |  | Climbers used for access have hand support for use while climbing |  |
|  |  |  | Rungs and handrails have a diameter between .95 and 1.55 - Rungs are secured to prevent turning - Between .60” and 1.20” for toddlers |  |
|  |  |  | The stepping surface used for final access (for rung ladders, flexible components, arch climbers, etc.) is not above the designated play surface it serves. |  |
|  |  |  | Handrail height: Vertical distance between the edge of the step and the top surface of the handrail is: - Between 15” and 20” for toddlers - Between 22” and 26” for preschool age - Between 22” and 38” for school age - Begins with the first step |  |
|  |  |  | Stairs more than 48” above the surface have barriers - Measure height of highest step in a series of steps to determine |  |
|  |  |  | PLATFORMS |  |
|  |  |  | Maximum difference in height between stepped platforms are: - 7” for toddlers - 12” for preschool age children - 18” for school age children - If difference is more than noted, an access component is needed |  |
|  |  |  | Openings between platforms are not a head entrapment hazard |  |
|  |  |  | If the space between platforms exceeds 9” and the height of the lower platform exceeds 30” for preschool or 48” for school age, in-fill is used to reduce the space to less than 3.5” |  |
|  |  |  | Platforms intended for toddlers are no more than 32” above the protective surfacing |  |

School:__________________________	     Play site: _______________________________ 	Date: ___/____/____

| C | NC | N/A | GUARDRAILS AND PROTECTIVE BARRIERS | Comments |
| --- | --- | --- | --- | --- |
|  |  |  | Openings between uprights and between barrier and platform surface are not a head entrapment hazard |  |
|  |  |  | Guardrails and barriers: - Designed to prevent falls off the platform - Discourage climbing on the barrier - Aid supervision |  |
|  |  |  | Guardrails/barriers completely surround elevated platform except for entrance and exit opening - 15” maximum to access play event - If more than 15”, must have one top rail of a guardrail - Stairs, ramps and upper body equipment are exempt from this requirement |  |
|  |  |  | Solid panels used as infill have transparent areas to facilitate supervision and permit viewing from platform |  |
|  |  |  | Guardrails are provided for preschool age: - Elevated surface more than 20” but not over 30” above protective surface - Minimum height of top rail is 29” - Maximum height of lower rail is 23” |  |
|  |  |  | Guardrails are provided for school age: - Elevated surface more than 30” but not over 48” above protective surface - Minimum height of top rail is 38” - Maximum height of lower rail is 28” |  |
|  |  |  | Barriers are provided for toddlers: - Elevated surface more than 18” above protective surface - Minimum height of barrier is 24” - Guardrails are not recommended |  |
|  |  |  | Barriers are provided for preschool age: - Elevated surface more than 30” above protective surface - Minimum height of barrier is 29” |  |
|  |  |  | Barriers are provided for school age: - Elevated surface more than 48” above protective surface - Minimum height of barrier is 38” |  |

School:__________________________	     Play site: _______________________________ 	Date: ___/____/____

| C | NC | N/A | DURABILITY & FINISH | Comments |
| --- | --- | --- | --- | --- |
|  |  |  | Metal surfaces painted or treated to prevent rust - Bare metal slides or platform surfaces are to be avoided unless away from the sun or covered |  |
|  |  |  | Painted surfaces are in good shape and not chipped |  |
|  |  |  | Wood structures or play elements are in good condition - Wood support posts are undamaged below fall material. |  |
|  |  |  | Plastic materials and components are in good condition |  |
|  |  |  | HARDWARE |  |
|  |  |  | All nuts, bolts, pins, lock washers and other connectors are present and secure |  |
|  |  |  | All posts are capped and sealed |  |
|  |  |  | Hardware in moving joints are secure |  |
|  |  |  | All S-hooks are closed so there is no gap or space greater than .04” or wide enough to admit a dime. |  |
|  |  |  | PROJECTIONS, PROTRUSIONS AND PINCH POINTS |  |
|  |  |  | Equipment is free of sharp points and corners are rounded. Caps or plugs cover exposed open ends of tubing. Wood parts are smooth and free of splinters. |  |
|  |  |  | Protrusions and projections do not extend beyond the face of any of the 3 gauges nor increase in the direction away from the surrounding surface nor project beyond the face of the nut more than 2 full threads. |  |
|  |  |  | Protrusions on swing assemblies: - Using the swing test gauge, no bolt or component protrudes beyond it (1/8”). |  |
|  |  |  | Connecting devices on swings: - S-hooks, C-hooks, pelican hooks, etc. are closed so there is no gap or space greater than .04” - No portion of the lower loop of an S-hook projects beyond the vertical boundary of the upper loop - Upper loop of S-hook may align, partially overlap or completely overlap without extending past the connector body - S-hook lower loop aligns with connector body and does not overlap. |  |
|  |  |  | Protrusions on slides: - Not great than 1/8” - Slide has a smooth, continuous sliding surface without any gaps or spaces that may create an entanglement hazard |  |
|  |  |  | No accessible pinch, crush or shearing points. May be caused by components moving relative to each other or to a fixed component. |  |

School:__________________________	     Play site: _______________________________ 	Date: ___/____/____

| C | NC | N/A | LABELS |  | Comments |
| --- | --- | --- | --- | --- | --- |
|  |  |  | Signs and/or stickers indicate the following: |  |  |
|  |  |  | - age appropriateness of the equipment |  |  |
|  |  |  | - supervision recommendation |  |  |
|  |  |  | - hazard of play equipment located over hard surfaces |  |  |
|  |  |  | - equipment manufacturer |  |  |
|  |  |  | - hot play surfaces warning |  |  |
|  |  |  | - hazards of drawstrings, accessories and other things worn around the neck |  |  |
|  |  |  | - removal of helmets (i.e. bike helmets) |  |  |
|  |  |  | ENTRAPMENT |  |  |
|  |  |  | All openings on equipment are less than 3.5” or greater than 9” - Use the head entrapment tools |  |  |
|  |  |  | Partially bound openings do not present an entrapment hazard - Use the partially bound opening template (“fish probe”) - Partially bound openings less than 24” above the surface are exempt |  |  |
|  |  |  | Angles formed by adjacent components are greater than 55 degrees - Unless lower leg is horizontal or projects downward |  |  |
|  |  |  | Flexible components (i.e. rope, cable or chains) are secured at both ends and not capable of being looped back on itself. |  |  |
|  |  |  | Flexible components suspended between play units are not located in high traffic areas. |  |  |

School:__________________________	     Play site: _______________________________ 	Date: ___/____/____

| C | NC | N/A | CLIMBING AND UPPER BODY EQUIPMENT | Comments |
| --- | --- | --- | --- | --- |
|  |  |  | Maximum fall height for free standing and composite climbing structures for toddlers is 32” |  |
|  |  |  | Climbers do not have bars or other structural components in the interior of the structure onto which a child may fall from a height of more than 18” |  |
|  |  |  | Equipment allows children to descend as easily as they ascend. Especially important for preschool equipment. |  |
|  |  |  | Layout of equipment does not facilitate climbing to top support bars of upper body equipment nor interfere with movement on adjacent structures. |  |
|  |  |  | Spacing between components does not present head entrapment hazards |  |
|  |  |  | Flexible climbing equipment: - Is not the sole means of access to components intended for use by 2-5 year olds or toddlers - Anchoring device is beneath protective surfacing - Perimeter of net opening should be less than 17” or greater than 28” - Free standing flexible climbers are not recommended for toddlers or preschool age |  |
|  |  |  | Arch climber: - Is not the sole means of access to components intended for use by 2-5 year olds - Free standing arch climbers are not allowed on preschool or toddler playgrounds - Rungs are .95” to 1.55” in diameter |  |
|  |  |  | Horizontal ladders and overhead rings: - Distance from edge of take-off/landing structure to first handhold is no greater than 10” and not directly above platform when platform is the access - Rungs on ladders intended for 4-5 year olds are parallel and evenly spaced - Space between adjacent rungs – no more than 12” for preschool and 15” for school age (does not apply to rings if rings swing) - Maximum length of chains for overhead rings is 7 inches. |  |
|  |  |  | Horizontal ladders and overhead rings: - Maximum height from center of grasping device to surfacing is 60” for preschool or 84” for school age - Maximum height for take-off/landing structure is 18” for preschool and 36” for school age |  |

School:__________________________	     Play site: _______________________________ 	Date: ___/____/____

| C | NC | N/A | EQUIPMENT | Comments |
| --- | --- | --- | --- | --- |
|  |  |  | Balance beams: - Maximum height 12” for preschool - Maximum height 16” for school age - Not recommended for toddlers |  |
|  |  |  | Climbing ropes: - Secure at both ends and not able to make a loop with an inside perimeter greater than 5” |  |
|  |  |  | Seesaws: - Prohibited for toddlers and preschool unless equipped with a spring centering device - Fulcrum is free of pinch and crush points - Shock absorbing material under seats to minimize impact with ground - Handholds do not protrude beyond sides of seat - Handgrips are 3” minimum if intended to be gripped by one hand and 6” if intended to be gripped by 2 hands - Maximum 60” height of seat above protective surface |  |
|  |  |  | Log rolls: - Prohibited for use by children less than 5 years - 18” above fall surface material - Hand gripping components for use; .95” to 1.55” in diameter |  |
|  |  |  | Track Rides: - Prohibited for use by children less than 5 years - Hand grip is 64” minimum and 78” maximum above the surface material - Landings (if used) have a minimum 36” length and 32” minimum width - Center to center distance between adjacent tracks are 48” minimum |  |
|  |  |  | Sliding poles: - Prohibited on equipment for toddlers and preschool age - Continuous with no welds or seams - Horizontal distance between platform edge and pole is between 18” and 20” - Pole extends at least 60” above the platform - Pole diameter is 1.9” maximum - Guardrail or barrier at pole access has an opening of 15” maximum |  |

School:__________________________	     Play site: _______________________________ 	Date: ___/____/____

| C | NC | N/A | SLIDES | Comments |
| --- | --- | --- | --- | --- |
|  |  |  | Depth of transition platform is 14” or greater - Width equal to or greater than width of sliding chute - Depth for equipment intended for toddlers is 19” |  |
|  |  |  | Guardrail or hood present to channel user into a sitting position |  |
|  |  |  | Slide entrance minimizes likelihood of entanglements |  |
|  |  |  | Straight slides with flat open chutes have 4”minimum sides for entire length of sliding surface |  |
|  |  |  | Slide chute is continuous without any gaps. Roller slides are excluded from this requirement. |  |
|  |  |  | 21” of clearance surrounds the slide chute |  |
|  |  |  | Metal slides are shaded to prevent burns from direct sun |  |
|  |  |  | Exit region of slide is: - 11” above protective surfacing for slides no more than 4 feet in height - Between 7” and 15” above protective surfacing for slides over 4 feet in height - Toddler slides – no more than 6” above protective surfacing |  |
|  |  |  | Embankment slides: - Maximum height of 12” above the ground - Chute entrance minimizes use by skateboards, bikes, etc. |  |
|  |  |  | Spiral slides: - One turn (360 degrees) or less for toddlers and preschool |  |
|  |  |  | Tube slides: - Minimum internal diameter no less than 23” |  |
|  |  |  | Roller slides: - Space between adjacent rollers and between ends of rollers and stationary structure are less than 3/16” |  |

School:__________________________	     Play site: _______________________________ 	Date: ___/____/____

| C | NC | N/A | SWINGS | Comments |
| --- | --- | --- | --- | --- |
|  |  |  | Prohibited: - Swings attached to composite structures - Animal figure swings - Rope swings - Swinging dual exercise rings and trapeze bars |  |
|  |  |  | S-hooks are closed with no gap or space great than .04”. Gap cannot admit a dime. |  |
|  |  |  | Swing structure discourages climbing. A-frame supports do not have cross-bars |  |
|  |  |  | 2 swings per bay maximum |  |
|  |  |  | Swing hangers at pivot spaced no less than 20” apart |  |
|  |  |  | **Swing spacing – toddler (bucket): - 20” between swing and support structure - 20” between swings - No less than 24” underside of occupied swing to surface - Located in a separate bay from other swings - Pivot points are between 47” and 96” above the protective surfacing |  |
|  |  |  | **Swing spacing – preschool: - 30” between swing and support structure - 24” between swings - No less than 12” underside of occupied swing to surface |  |
|  |  |  | **Swing spacing – school age: - 30” between swing and support structure - 24” between swings - No less than 12” underside of occupied swing to surface |  |
|  |  |  | Multi-Axis Tire Swing: - Minimum clearance between seating surface and upright of support structure is 30” when tire is in a position closest to the support structure. - No less than 12” underside of occupied swing to surface |  |
|  |  |  | To be accessible and meet ADA requirements, the seat of a swing must be between 11” and 24” above the surface material |  |

** Measurements taken at 60” above protective surfacing

School:__________________________	     Play site: _______________________________ 	Date: ___/____/____

| C | NC | N/A | ROTATING & ROCKING EQUIPMENT | Comments |
| --- | --- | --- | --- | --- |
|  |  |  | Merry-Go-Rounds and Whirls: - Components do not extend beyond the perimeter of the platform - Underside of platform is no less than 9” above the surface - No openings between the axis and periphery that permit a 5/16 rod to penetrate - No accessible shearing or crushing mechanism in the undercarriage - Maximum height for standing/sitting surface is 14” above surface for preschool and 18” for school age - Hand grips have a diameter of 0.95” to 1.55” - Not recommended for playgrounds intended for toddlers |  |
|  |  |  | Spring Rockers: - Springs minimize possibility of pinching hands or feet - Each seating position has handgrips and footrests - Handgrips are 3” minimum if intended to be gripped by one hand and 6” if intended to be gripped by 2 hands - Seat height is between 12” and 16” for toddlers - Seat height is between 14” and 28” for preschool - Hand grips have a diameter of 0.60” to 1.20” for toddlers - Hand grips have a diameter of 0.95” to 1.55” for preschool and school age |  |